DUAL-CORE OPTICAL MOTION
TESTBED

FIRMWARE, HOST SOFTWARE & RESULTS

STM32H747 MotionIPC, live telemetry, automated characterization, and measured operation

| Prepared by | Herder Elektronische Systemen |
| --- | --- |
| Report scope | Dual-core firmware architecture, host GUI, telemetry integrity, test automation, and measured motion performance |
| Evidence basis | Source code, engineering plots, project photographs, CAD/PCB artifacts, and recorded demonstrations |
| Revision | 4.0 - 5 August 2026 |

DOCUMENT PURPOSE This report documents the software-controlled testbed: target-side motion control, dual-core interprocessor communication, operator software, data products, safeguards, and measured behavior. Hardware and PCB implementation are intentionally contained in the companion platform report.

# 1. System Role and Delivered Capability

The system combines a dual-core STM32H747 target with a host application that commands motion, displays live encoder and speed telemetry, executes repeatable tests, and writes analysis-ready records. The public evidence package separates control-plane messaging from high-rate motion-data movement and preserves the distinction between demonstrated behavior and recommended release actions.

Cortex-M7 and Cortex-M4 workloads connected through OpenAMP/RPMsg and a custom AXI-SRAM shared ring.

Live host console for connection, scripted test selection, telemetry plots, CSV logging, and metadata capture.

Measured low-speed tracking, constant-speed error, step response, and closed-loop frequency-response evidence.

Fault, heartbeat, sequence-gap, dropped-record, and ring-status observability.

Figure 1. Live host telemetry console during an automated characterization sequence.

# 2. Dual-Core Firmware Architecture

Figure 2. Dual-core architecture with physically separated control-message and shared-ring paths.

| Interface | Verified role | Representative parameter |
| --- | --- | --- |
| OpenAMP / RPMsg | Low-rate command and status coordination | 44-byte application message |
| AXI-SRAM ring | High-rate aligned motion records | 64-byte records; 4,096 nominal entries |
| M7 producer | Motion sequencing and sample publication | 100 Hz sample production in the reviewed package |
| M4 consumer | Bounded service and accounting | Up to 128 records per service burst |

# 3. Shared Memory, Timing, and Integrity

Figure 3. MotionIPC shared-memory allocation and the engineering review finding on nominal ring capacity.

OpenAMP resources occupy D3 SRAM while the custom producer/consumer ring is allocated in AXI SRAM.

Records are aligned to cache-line boundaries and include sequence, overflow, and heartbeat information.

Engineering review identified that 4,096 records plus metadata exceed a 256 KiB allocation by approximately one record; release should use 4,095 entries or enlarge the reserved region.

The target reports independent heartbeat and fault state so host-side silence is distinguishable from target-side failure.

# 4. Host Application and Test Workflow

Figure 4. Host console view showing live position, commanded/measured speed, and raw encoder increments.

| Workflow stage | Operator action | Recorded output |
| --- | --- | --- |
| Connect | Select serial port and baud rate | Target identity and connection state |
| Configure | Select firmware and test definition | Run metadata and configuration |
| Execute | Start automated sequence | Live plots and telemetry rows |
| Review | Open generated outputs | CSV, figures, logs, and summary metrics |

# 5. Low-Speed Tracking Results

Figure 5. Speed-tracking summary showing the usable ultra-slow range and high-speed ceiling.

| Commanded speed (deg/s) | Measured speed (deg/s) | Interpretation |
| --- | --- | --- |
| 0.002 | 0.001818 | Trackable near the lower practical limit |
| 0.003 | 0.003214 | Best agreement region |
| 0.005 | 0.004676 | Strong low-speed tracking |
| 0.010 | 0.008378 | Usable with increasing bias |
| 0.050-0.100 | ~0.016 plateau | Configuration-limited ceiling |

# 6. Constant-Speed and Step-Response Characterization

Figure 6. Extended constant-speed tracking and Allan-style stability summary.

Figure 7. Step-response metrics for RMS following error, worst following error, gain, and position-area measures.

# 7. Closed-Loop Frequency Response

Figure 8. Closed-loop response estimate across the tested 0.33-2 Hz range.

The available evidence supports reporting response across the tested frequencies. It does not, by itself, establish a formal -3 dB bandwidth because the plotted data do not demonstrate a complete crossing with sufficient surrounding points.

# 8. Safety, Reliability, and Release Controls

| Control | Implemented behavior | Release consideration |
| --- | --- | --- |
| Host heartbeat | Periodic host presence message | Target deadman should remain enabled |
| Pulse-rate cap | 25,000 pulses/s maximum | Verify against final drivetrain and driver |
| Backlog bound | Maximum 25 pulses emitted per loop | Prevents unlimited catch-up bursts |
| Telemetry limits | Selectable 10/20/50/100 Hz | Full ASCII rows may constrain sustainable rate |
| Fault accounting | Sequence gaps, drops, heartbeat, fault counters | Expose in final operator status panel |

At 115,200 baud, full 24-field ASCII telemetry rows cannot be assumed to sustain 100 Hz under all conditions. The configured option should be treated separately from demonstrated end-to-end serial throughput; 50-60 Hz is a more realistic full-row operating range without a binary protocol or higher link rate.

# 9. Limitations and Document Control

| Item | Controlled statement |
| --- | --- |
| Prototype maturity | No formal shock, vibration, thermal, EMC, or long-duration certification is claimed. |
| MotionIPC package | Reviewed v0.1 sample producer is 100 Hz and should not be represented as the final integrated motor-control workload without the corresponding integrated revision. |
| Bandwidth claim | Tested response is reported; formal bandwidth is not claimed. |
| Release package | Preserve tagged source, host build, test definitions, and representative datasets together. |

## Attribution

Herder Elektronische Systemen developed the public-facing engineering package and documents the dual-core firmware integration, host software, test workflows, and measured evidence represented here. No third-party affiliation or endorsement is claimed.

## Revision record

| Revision | Date | Description |
| --- | --- | --- |
| 4.0 | 2026-08-05 | Rebuilt from scratch with restored GUI, architecture, and measured-result imagery; removed inherited layout defects and preserved evidence boundaries. |